A Dial-Up Network of UNIXTM Systems

D. A. Nowitz

M. E. Lesk

Bell Laboratories

Murray Hill, New Jersey 07974

_#A_#B_#S_#T_#R_#A_#C_#T

A network of over eighty UNIX* computer  sys-

tems has been established using the telephone sys-

tem as its primary communication medium.  The net-

work  was designed to meet the growing demands for

software distribution and exchange.   Some  advan-

tages of our design are:

-    The startup cost is low.  A system needs only

a  dial-up  port,  but systems with automatic

calling units have much more flexibility.

-    No operating system changes are  required  to

install or use the system.

-    The communication is basically  over  dial-up

lines, however, hardwired communication lines

can be used to increase speed.

-    The command for  sending/receiving  files  is

simple to use.

Keywords: networks, communications,  software

distribution, software maintenance

August 18, 1978

__________________________

*UNIX is a Trademark of Bell Laboratories.

A Dial-Up Network of UNIXTM Systems

D. A. Nowitz

M. E. Lesk

Bell Laboratories

Murray Hill, New Jersey 07974

_#1.  _#P_#u_#r_#p_#o_#s_#e

The widespread use of the UNIX* system ritchie thompson

bstj  1978 within Bell Laboratories has produced problems of

software  distribution  and  maintenance.   A   conventional

mechanism  was set up to distribute the operating system and

associated programs from  a  central  site  to  the  various

users.   However  this  mechanism  alone  does  not meet all

software distribution needs.   Remote  sites  generate  much

software  and  must  transmit  it to other sites.  Some UNIX

systems are themselves central sites for redistribution of a

particular  specialized  utility, such as the Switching Con-

trol Center System.   Other  sites  have  particular,  often

long-distance   needs   for   software  exchange;  switching

research, for example, is carried on in  New  Jersey,  Illi-

nois,  Ohio,  and  Colorado.   In  addition, general purpose

utility programs are written at all UNIX system sites.   The

UNIX  system is modified and enhanced by many people in many

places and it would be  very  constricting  to  deliver  new

software in a one-way stream without any alternative for the

user sites to respond with changes of their own.

Straightforward software distribution is only  part  of

the  problem.   A large project may exceed the capacity of a

single computer and several machines may be used by the  one

group of people.  It then becomes necessary for them to pass

messages, data and other information back an  forth  between

computers.

Several groups with similar problems, both  inside  and

outside  of  Bell  Laboratories,  have  constructed networks

built of hardwired connections only.   dolotta  mashey  1978

bstj  network unix system chesson Our network, however, uses

both dial-up and hardwired connections so that  service  can

be provided to as many sites as possible.

__________________________

*UNIX is a Trademark of Bell Laboratories.

- 2 -                      Nowitz

_#2.  _#D_#e_#s_#i_#g_#n _#G_#o_#a_#l_#s

Although some of our machines are  connected  directly,

others  can  only  communicate over low-speed dial-up lines.

Since the dial-up  lines  are  often  unavailable  and  file

transfers  may take considerable time, we spool all work and

transmit in the background.  We also had to adapt to a  com-

munity  of  systems  which  are  independently  operated and

resistant to suggestions that they should all buy particular

hardware  or  install  particular operating system modifica-

tions.  Therefore, we make  minimal  demands  on  the  local

sites  in  the  network.   Our  implementation  requires  no

operating system changes; in  fact,  the  transfer  programs

look  like  any  other  user entering the system through the

normal dial-up login ports, and obeying all local protection

rules.

We distinguish ``active'' and  ``passive''  systems  on

the  network.  Active systems have an automatic calling unit

or a hardwired line to another system, and  can  initiate  a

connection.   Passive  systems  do  not have the hardware to

initiate a connection.  However, an  active  system  can  be

assigned  the  job  of calling passive systems and executing

work found there; this makes a passive system the functional

equivalent  of  an  active  system, except for an additional

delay while it waits to be polled.  Also, people  frequently

log into active systems and request copying from one passive

system to another.  This requires two telephone  calls,  but

even so, it is faster than mailing tapes.

Where convenient, we use hardwired communication lines.

These  permit  much  faster transmission and multiplexing of

the communications link.  Dial-up connections  are  made  at

either 300 or 1200 baud; hardwired connections are asynchro-

nous up to 9600 baud and might run even faster  on  special-

purpose  communications  hardware.   fraser spider 1974 ieee

fraser channel network datamation 1975 Thus,  systems  typi-

cally  join  our  network  first as passive systems and when

they find the service more important, they acquire automatic

calling  units  and  become active systems; eventually, they

may install high-speed links  to  particular  machines  with

which  they  handle  a  great deal of traffic.  At no point,

however, must users change their programs or procedures.

The basic operation of  the  network  is  very  simple.

Each  participating  system  has a spool directory, in which

work to be done (files to be moved, or commands to  be  exe-

cuted remotely) is stored.  A standard program, _#u_#u_#c_#i_#c_#o, per-

forms all transfers.  This program starts by  identifying  a

particular  communication  channel  to  a remote system with

which it will hold a conversation.  _#U_#u_#c_#i_#c_#o  then  selects  a

device  and establishes the connection, logs onto the remote

machine and starts the _#u_#u_#c_#i_#c_#o program on the remote machine.

Once  two  of these programs are connected, they first agree

- 3 -                      Nowitz

on a line protocol, and then start  exchanging  work.   Each

program  in turn, beginning with the calling (active system)

program, transmits everything it needs, and  then  asks  the

other  what  it wants done.  Eventually neither has any more

work, and both exit.

In this way, all services are available from all sites;

passive  sites,  however, must wait until called.  A variety

of protocols may be used; this conforms to  the  real,  non-

standard  world.   As long as the caller and called programs

have a protocol in common, they can  communicate.   Further-

more, each caller knows the hours when each destination sys-

tem should be called.  If a destination is unavailable,  the

data intended for it remain in the spool directory until the

destination machine can be reached.

The implementation of this  Bell  Laboratories  network

between  independent  sites,  all of which store proprietary

programs and data,  illustratives  the  pervasive  need  for

security and administrative controls over file access.  Each

site, in configuring its programs and system  files,  limits

and monitors transmission.  In order to access a file a user

needs access permission for the machine  that  contains  the

file  and  access  permission  for the file itself.  This is

achieved by first requiring the user to use his password  to

log  into  his local machine and then his local machine logs

into the remote machine whose files are to be accessed.   In

addition,  records  are  kept identifying all files that are

moved into and out of the local system, and how the  reques-

tor  of  such  accesses  identified himself.  Some sites may

arrange to permit users only to call up and request work  to

be  done;  the calling users are then called back before the

work is actually done.  It is then possible to  verify  that

the  request is legitimate from the standpoint of the target

system, as well as  the  originating  system.   Furthermore,

because  of the call-back, no site can masquerade as another

even if it knows all the necessary passwords.

Each machine can optionally maintain a  sequence  count

for  conversations with other machines and require a verifi-

cation of the count  at  the  start  of  each  conversation.

Thus,  even  if  call  back  is  not  in  use,  a successful

masquerade requires the calling party to present the correct

sequence  number.   A  would-be  impersonator  must not just

steal the correct phone number, user name, and password, but

also  the  sequence  count,  and  must  call in sufficiently

promptly to precede the next legitimate request from  either

side.   Even a successful masquerade will be detected on the

next correct conversation.

_#3.  _#P_#r_#o_#c_#e_#s_#s_#i_#n_#g

The user has two commands which set up  communications,

_#u_#u_#c_#p  to  set  up  file  copying,  and _#u_#u_#x to set up command

- 4 -                      Nowitz

execution where  some  of  the  required  resources  (system

and/or  files)  are not on the local machine.  Each of these

commands will put work and data files into the spool  direc-

tory  for  execution  by  _#u_#u_#c_#p  daemons.  Figure 1 shows the

major blocks of the file transfer process.

_#F_#i_#l_#e _#C_#o_#p_#y

The _#u_#u_#c_#i_#c_#o program is used to  perform  all  communica-

tions  between  the  two systems.  It performs the following

functions:

-  Scan the spool directory for work.

-  Place a call to a remote system.

-  Negotiate a line protocol to be used.

-  Start program _#u_#u_#c_#i_#c_#o on the remote system.

-  Execute all requests from both systems.

-  Log work requests and work completions.

_#U_#u_#c_#i_#c_#o may be started in several ways;

a)   by a system daemon,

b)   by one of the _#u_#u_#c_#p or _#u_#u_#x programs,

c)   by a remote system.

_#S_#c_#a_#n _#F_#o_#r _#W_#o_#r_#k

The file names in the spool directory  are  constructed

to  allow  the  daemon programs (_#u_#u_#c_#i_#c_#o, _#u_#u_#x_#q_#t) to determine

the files they should look  at,  the  remote  machines  they

should  call and the order in which the files for a particu-

lar remote machine should be processed.

_#C_#a_#l_#l _#R_#e_#m_#o_#t_#e _#S_#y_#s_#t_#e_#m

The call is made using information from  several  files

which reside in the uucp program directory.  At the start of

the call process, a lock is set on the system  being  called

so that another call will not be attempted at the same time.

The system name is found in a  ``systems''  file.   The

information contained for each system is:

[1]  system name,

- 5 -                      Nowitz

[2]  times to call the system (days-of-week and  times-

of-day),

[3]  device or device type to be used for call,

[4]  line speed,

[5]  phone number,

[6]  login information (multiple fields).

The time field is checked against the present  time  to

see  if  the call should be made.  The _#p_#h_#o_#n_#e _#n_#u_#m_#b_#e_#r may con-

tain abbreviations  (e.g.  ``nyc'',  ``boston'')  which  get

translated  into dial sequences using a ``dial-codes'' file.

This permits the same ``phone number'' to be stored at every

site,  despite  local  variations  in telephone services and

dialing conventions.

A ``devices'' file is scanned using fields [3] and  [4]

from  the  ``systems''  file to find an available device for

the connection.  The program  will  try  all  devices  which

satisfy  [3]  and [4] until a connection is made, or no more

devices can be tried.  If a non-multiplexable device is suc-

cessfully  opened,  a  lock  file is created so that another

copy of _#u_#u_#c_#i_#c_#o will not try to use it.  If the connection is

complete,  the  _#l_#o_#g_#i_#n  _#i_#n_#f_#o_#r_#m_#a_#t_#i_#o_#n  is  used to log into the

remote system.  Then a command is sent to the remote  system

to  start  the _#u_#u_#c_#i_#c_#o program.  The conversation between the

two _#u_#u_#c_#i_#c_#o programs begins with a handshake started  by  the

called, _#S_#L_#A_#V_#E, system.  The _#S_#L_#A_#V_#E sends a message to let the

_#M_#A_#S_#T_#E_#R know it is ready to receive the system identification

and  conversation  sequence  number.   The response from the

_#M_#A_#S_#T_#E_#R is verified by the _#S_#L_#A_#V_#E and if acceptable,  protocol

selection begins.

_#L_#i_#n_#e _#P_#r_#o_#t_#o_#c_#o_#l _#S_#e_#l_#e_#c_#t_#i_#o_#n

The remote system sends a message

P_#p_#r_#o_#t_#o-_#l_#i_#s_#t

where _#p_#r_#o_#t_#o-_#l_#i_#s_#t is a string of characters, each  represent-

ing  a line protocol.  The calling program checks the proto-

list for a letter corresponding to an available line  proto-

col  and  returns  a _#u_#s_#e-_#p_#r_#o_#t_#o_#c_#o_#l message.  The _#u_#s_#e-_#p_#r_#o_#t_#o_#c_#o_#l

message is

U_#c_#o_#d_#e

where code is either a one character protocol letter or a  _#N

which means there is no common protocol.

Greg Chesson designed and implemented the standard line

- 6 -                      Nowitz

protocol  used by the uucp transmission program.  Other pro-

tocols may be added by individual installations.

_#W_#o_#r_#k _#P_#r_#o_#c_#e_#s_#s_#i_#n_#g

During processing, one program is the  _#M_#A_#S_#T_#E_#R  and  the

other  is _#S_#L_#A_#V_#E.  Initially, the calling program is the _#M_#A_#S_#-

_#T_#E_#R.  These roles may switch one or more  times  during  the

conversation.

There are four messages used during the  work  process-

ing,  each  specified by the first character of the message.

They are

center; c l.  S       send a file, R       receive  a  file,

C       copy complete, H       hangup.

The _#M_#A_#S_#T_#E_#R will send _#R or _#S messages until all work from the

spool  directory  is  complete,  at which point an _#H message

will be sent.  The _#S_#L_#A_#V_#E will reply with _#S_#Y, _#S_#N, _#R_#Y, _#R_#N, _#H_#Y,

_#H_#N, corresponding to _#y_#e_#s or _#n_#o for each request.

The send and receive replies are based on permission to

access  the  requested  file/directory.   After each file is

copied into the spool directory of the receiving  system,  a

copy-complete  message  is sent by the receiver of the file.

The message _#C_#Y will be sent if the UNIX _#c_#p command, used  to

copy  from the spool directory, is successful.  Otherwise, a

_#C_#N message is sent.  The requests and results are logged  on

both  systems,  and,  if requested, mail is sent to the user

reporting completion (or the user can request status  infor-

mation from the log program at any time).

The hangup response is determined by the _#S_#L_#A_#V_#E  program

by  a  work  scan  of  the spool directory.  If work for the

remote system exists in the _#S_#L_#A_#V_#E'_#s spool  directory,  a  _#H_#N

message  is  sent and the programs switch roles.  If no work

exists, an _#H_#Y response is sent.

A sample conversation is shown in Figure 2.

_#C_#o_#n_#v_#e_#r_#s_#a_#t_#i_#o_#n _#T_#e_#r_#m_#i_#n_#a_#t_#i_#o_#n

When a _#H_#Y message is  received  by  the  _#M_#A_#S_#T_#E_#R  it  is

echoed  back  to the _#S_#L_#A_#V_#E and the protocols are turned off.

Each program sends a final "OO" message to the other.

_#4.  _#P_#r_#e_#s_#e_#n_#t _#U_#s_#e_#s

One application of this software is remote mail.   Nor-

mally,  a  UNIX system user writes ``mail dan'' to send mail

to user ``dan''.  By writing ``mail usg!dan''  the  mail  is

sent to user ``dan'' on system ``usg''.

- 7 -                      Nowitz

The primary uses of our network to date  have  been  in

software  maintenance.   Relatively  few of the bytes passed

between systems are intended for people to  read.   Instead,

new  programs  (or  new  versions  of  programs) are sent to

users, and potential bugs are returned  to  authors.   Aaron

Cohen  has  implemented  a ``stockroom'' which allows remote

users to call in and request software.  He keeps  a  ``stock

list''  of  available programs, and new bug fixes and utili-

ties are added regularly.  In this  way,  users  can  always

obtain  the latest version of anything without bothering the

authors of the programs.  Although the stock list  is  main-

tained  on  a  particular system, the items in the stockroom

may be warehoused in many places; typically each program  is

distributed  from the home site of its author.  Where neces-

sary, uucp does remote-to-remote copies.

We also routinely retrieve test cases from  other  sys-

tems  to  determine  whether  errors  on  remote systems are

caused  by  local  misconfigurations  or  old  versions   of

software, or whether they are bugs that must be fixed at the

home site.  This helps identify errors rapidly.  For one set

of  test  programs  maintained  by  us, over 70% of the bugs

reported from remote sites were due  to  old  software,  and

were fixed merely by distributing the current version.

Another application of the network for software mainte-

nance is to compare files on two different machines.  A very

useful utility  on  one  machine  has  been  Doug  McIlroy's

``diff'' program which compares two text files and indicates

the differences, line by line, between them.   hunt  mcilroy

file  Only lines which are not identical are printed.  Simi-

larly, the program  ``uudiff''  compares  files  (or  direc-

tories) on two machines.  One of these directories may be on

a passive system.  The ``uudiff'' program is set up to  work

similarly  to the inter-system mail, but it is slightly more

complicated.

To avoid moving  large  numbers  of  usually  identical

files, _#u_#u_#d_#i_#f_#f computes file checksums on each side, and only

moves files that are different for detailed comparison.  For

large  files, this process can be iterated; checksums can be

computed for each line, and only those lines that  are  dif-

ferent actually moved.

The ``uux''  command  has  been  useful  for  providing

remote  output.   There  are some machines which do not have

hard-copy devices, but which are connected  over  9600  baud

communication lines to machines with printers.  The _#u_#u_#x com-

mand allows the formatting of  the  printout  on  the  local

machine  and  printing  on the remote machine using standard

UNIX command programs.

- 8 -                      Nowitz

_#5.  _#P_#e_#r_#f_#o_#r_#m_#a_#n_#c_#e

Throughput,  of  course,  is  primarily  dependent   on

transmission   speed.    The  table  below  shows  the  real

throughput of characters on communication links of different

speeds.   These  numbers  represent actual data transferred;

they do not include bytes used by the line protocol for data

validation  such  as  checksums and messages.  At the higher

speeds, contention for the processors on both ends  prevents

the  network from driving the line full speed.  The range of

speeds represents the difference  between  light  and  heavy

loads  on  the  two  systems.   If desired, operating system

modifications can be installed that permit full use of  even

very fast links.

center; c  c  n  n.   Nominal  speed   Characters/sec.   300

baud        27  1200 baud       100-110 9600 baud       200-

850

In addition to the transfer time, there is some overhead for

making the connection and logging in ranging from 15 seconds

to 1 minute.  Even at 300 baud,  however,  a  typical  5,000

byte  source  program  can  be  transferred  in four minutes

instead of the 2 days that might be required to mail a tape.

Traffic  between  systems  is  variable.   Between  two

closely  related  systems,  we observed 20 files moved and 5

remote commands executed in a typical day.   A  more  normal

traffic out of a single system would be around a dozen files

per day.

The total number of sites at present in the  main  net-

work  is  82,  which  includes most of the Bell Laboratories

full-size machines which  run  the  UNIX  operating  system.

Geographically,   the  machines  range  from  Andover,  Mas-

sachusetts to Denver, Colorado.

Uucp has also been used to set up another network which

connects  a  group  of systems in operational sites with the

home site.  The two networks touch at  one  Bell  Labs  com-

puter.

_#6.  _#F_#u_#r_#t_#h_#e_#r _#G_#o_#a_#l_#s

Eventually, we would like to develop a full  system  of

remote  software  maintenance.   Conventional maintenance (a

support group which mails tapes) has many well-known  disad-

vantages.   brooks mythical man month 1975 There are distri-

bution errors and delays, resulting in old software  running

at remote sites and old bugs continually reappearing.  These

difficulties are aggravated when  there  are  100  different

small systems, instead of a few large ones.

The availability of  file  transfer  on  a  network  of

- 9 -                      Nowitz

compatible  operating systems makes it possible just to send

programs directly to the end  user  who  wants  them.   This

avoids  the  bottleneck  of negotiation and packaging in the

central support group.  The ``stockroom'' serves this  func-

tion for new utilities and fixes to old utilities.  However,

it is still likely that distributions will not be  sent  and

installed  as often as needed.  Users are justifiably suspi-

cious of the ``latest version'' that has just  arrived;  all

too  often it features the ``latest bug.'' What is needed is

to address both problems simultaneously:

1.   Send distributions whenever programs change.

2.   Have sufficient quality  control  so  that  users  will

install them.

To do this, we recommend systematic regression testing  both

on the distributing and receiving systems.  Acceptance test-

ing on the receiving systems can be  automated  and  permits

the  local system to ensure that its essential work can con-

tinue despite the constant installation of changes sent from

elsewhere.  The work of writing the test sequences should be

recovered in lower counseling and distribution costs.

Some slow-speed network services are also being  imple-

mented.   We  now  have  inter-system ``mail'' and ``diff,''

plus the many implied commands represented by ``uux.''  How-

ever,   we  still  need  inter-system  ``write''  (real-time

inter-user communication) and ``who'' (list of people logged

in on different systems).  A slow-speed network of this sort

may be very useful for speeding up counseling and education,

even if not fast enough for the distributed data base appli-

cations that attract many users to networks.  Effective  use

of  remote  execution  over  slow-speed lines, however, must

await the general installation of multiplexable channels  so

that long file transfers do not lock out short inquiries.

_#7.  _#L_#e_#s_#s_#o_#n_#s

The following is a summary of the lessons we learned in

building these programs.

1.   By starting your network in  a  way  that  requires  no

hardware or major operating system changes, you can get

going quickly.

2.   Support will follow use.  Since the network existed and

was  being  used,  system  maintainers were easily per-

suaded to help keep it operating, including  purchasing

additional hardware to speed traffic.

3.   Make the network commands  look  like  local  commands.

Our  users  have a resistance to learning anything new:

all the inter-system  commands  look  very  similar  to

- 10 -                     Nowitz

standard  UNIX  system commands so that little training

cost is involved.

4.   An initial  error  was  not  coordinating  enough  with

existing  communications projects: thus, the first ver-

sion of this network was restricted to  dial-up,  since

it  did  not support the various hardware links between

systems.  This has been fixed in the current system.

_#A_#c_#k_#n_#o_#w_#l_#e_#d_#g_#e_#m_#e_#n_#t_#s

We thank G. L. Chesson for his design  and  implementa-

tion  of the packet driver and protocol, and A. S. Cohen, J.

Lions, and P. F. Long for their suggestions and  assistance.

$LIST$