IDLE10.DOC

A long time ago the utility "NIGHT" was posted.  It blanked the screen

after a few minutes with no activity, and unblanked it upon a keypress

or a mouse movement or BIOS text output to the screen.

NIGHT had serious bugs.  NITE fixed some, but it blanks the screen by

the same method: switching to external video sync.  If there are no

external sync signals, there are no vertical blanks, no picture---and

no VBL interrupts.  But many operations in the ST rely on those VBL

interrupts.  Leaving the ST on for long periods with NIGHT active leads

to strange behavior and a need for a reboot.

Here is IDLE, an alternative to NITE.  It should not cause any problems,

since all it does is shift the video RAM pointer 32K below the normal

location.  Since that area is normally all zeros, setting the video

palette right will display it as all black.  IDLE checks to see that

that piece of RAM is all zeros, and if it is not, it leaves the video

pointer as it was, and reverses the colors instead.  The check for

nonzeros is done slowly, so strange things _might_ show on the screen

for up to 20 seconds.  But not to worry: IDLE does _not_ write to the

video RAM.

I have only tested IDLE with a monochrome monitor, but it is supposed

to work with color too.

- Moshe Braner

------------------------------------------

IDLE12.DOC

Here is (ahem) yet another version of IDLE, version 1.2.

(Version 1.1 had yet another bug.)

Reminder: IDLE 1.2 replaces NIGHT, NITE, and earlier versions of IDLE,

all of which had serious bugs.  IDLE blanks the screen after 6 minutes

of no activity (up from 3 in 1.1) and unblanks upon keyboard/mouse

input or BIOS text output.

All IDLE does is zero the palette AND shift the video RAM pointer 32K

below the normal location.  Since that area is normally all zeros,

zeroing the video palette will display it as all black.  IDLE checks

to see that that piece of RAM is all zeros, and if it is not, it leaves

the video pointer as it was, but still zeros the palette.  This has the

effect of blanking the screen in color mode, but only reversing black

and white in monochrome mode.  The check for nonzeros is done slowly,

so strange things _might_ show on the screen for up to 20 seconds.

But not to worry: IDLE does _not_ write to the video RAM.

IDLE can _still_ cause problems with programs that change the screen RAM

location *** without user input and after 6 minutes of no input. ***

Since such a program might later restore the screen RAM location to

_either_ what it was when changed (the blank area) (case A) _or_ what

it was before it was blanked (case B), depending on the program design,

I cannot fix IDLE to handle all possible cases.  (Should I make IDLE

crash the program on purpose in such a case?)  Currently IDLE avoids

further fiddling with the screen location after detecting such a change.

That means problems in case A if the program changes back to what it

_thinks_ is the original screen.

To customize the delay before blanking, change the LONG integer that is

located immediately after the branch instruction at the start of the program,

at an offset of 32 bytes from the start of the .PRG file, and followed

with the ASCII string "IDLE" in the next 4 bytes.  The default value is

72000 decimal (00011940 hex) for 6 minutes (6*60*200).

I have only tested IDLE with a monochrome monitor, but it is supposed

to work with color too, as described.

- Moshe Braner