See URL: <http://www.lila.com/nautilus> for the most up-to-date copy of

the Nautilus manual and for general information related to Nautilus.

----------------------------------------------------------------------------

Index of the Nautilus v1.6 Documentation:

* Introduction

* System Requirements

* Quick Start

* Compiling Nautilus

* Environment Variables

* Configuring Nautilus

* Running Nautilus

* Appendix A: Getting Nautilus

* Appendix B: Cryptography and Security

* Appendix C: Nautilus Key Exchange Protocol

* Appendix D: Man in the Middle (MITM) attack

----------------------------------------------------------------------------

"Indeed, it seems that the phrase `National Security' is now the

root password to the Constitution.  And as with any dishonest

superuser, the best user countermeasure is strong encryption."

-- Phil Karn

----------------------------------------------------------------------------

1. Introduction

What Nautilus Is

Nautilus is a program that lets you have encrypted voice telephone

conversations with your friends without needing any special equipment. All

you need is a standard personal computer (386/25 or faster PC with

Soundblaster compatible sound board, or Sun Sparcstation) and a high speed

modem. Its speech quality is reasonably good at 14.4 kbps and acceptable at

9600 or 7200 bps. 7200 bps is low enough to work fairly reliably even over

cellular telephones. A new coder was added for the v1.5 release which allows

Nautilus to use a 4800 bps data rate. This coder has better sound quality

than the 7200 bps coder and should work even more reliably on cellular

phones because of its lower bit rate, but it needs a fast CPU with

floating-point hardware in order to function. A 486DX2-66 is about the

minimum necessary for its use.

Telephones are a much less secure medium than most people realize. It is

relatively easy for anyone (not just law enforcement agencies) to eavesdrop

on your phone conversations, without needing access to your phone wiring. By

dialing into telephone switches using access codes stolen from dumpsters,

"phone phreaks" can monitor your calls from across the country. Even the FBI

has been wire-tapped using these methods. And government abuse of wiretaps

against political opponents and others is well known. With cellular

telephones, the situation is even worse. Anyone with a simple scanner or

even a 1970's-vintage television set can eavesdrop on cellular conversations

with no special skills or equipment. (This is because cellular phones

operate on frequencies formerly occupied by UHF TV channels 69 to 83, and

older TV sets can still pick up these frequencies).

Anyone with a need for privacy in their phone calls should consider using

Nautilus. Journalists talking with sources, lawyers talking to clients,

businesspeople discussing company secrets, and couples wanting to talk about

intimate matters all need privacy. You may need it too.

Nautilus is the first program of this type that we know of to be distributed

for free with source code. A few similar programs have been distributed

without source, so that their security cannot be independently examined. We

have serious reservations against using any security program distributed

without source, even if the author and distribution channel is trusted,

because programs of this type need to be reviewed carefully by as many

people as possible. Consider the famous Netscape 1.0 security bug (now fixed)

where the internal random number generator was seeded from the system clock

and other sources with just a few total bits of entropy. This meant that

crackers could read Netscape credit card transactions using only minutes of

computing time. External review would catch bugs like this very quickly. Who

knows how many other bugs of this type a program like Netscape might have?

Even Federally supported and supposedly 'secure' digital signatures (DSS)

has been proven to be able to leak private signatures in certain

implementations (Schneier, 1st ed. p 304). Remember, you are closer to

that company who's product you trust than they are to you.

Get It While You Can

Certain parts of the US Government appear to be working to ban civilian use

of cryptography whose keys are not accessible to the government. Shortly

after the notorious key-escrowed Clipper chip was first announced, the FBI

testified before Congress that it was intended as a completely voluntary

system. But the Electronic Privacy Information Center, using the Freedom of

Information Act, recently obtained a briefing sent by FBI director William

Sessions to the National Security Council, titled "Encryption: The Threat,

Applications, and Potential Solutions". The briefing was prepared by the

FBI, the NSA, and the Department of Justice, and said among other things:

Technical solutions, such as they are, will only work if they are

incorporated into *all* encryption products. To ensure that this

occurs, legislation mandating the use of Government-approved

encryption products or adherence to Government encryption criteria

is required.

Several other documents obtained at the same time reinforce the obvious

implications of this passage. Unless there has been a total policy reversal

at the upper levels of these security agencies since February 1993, we

conclude that there is a continuing desire on their part to make programs

like Nautilus illegal sometime in the future. We will have to stop further

development and distribution if that happens.

Recent events, like the Digital Telephony Act of 1994 give police agencies

much broader powers to conduct "legal" wiretaps (up to 1% of all American

phones) *without* a warrant. 500 million dollars has been budgeted by the

Federal Government to pay private and public companies which produce PBX's

and other switching equipment to produce new telephony equipment with

built-in digital remote wiretapping ability over the years of 1995-1997.

Although this same law which also "deregulates" the industry, the intent

to tap U.S. citizen's home, business and cellular phones is written in

black and white. And Attorney General Janet Reno has stated in CNN color

that she expects to be able to tap criminals, not the phone. This is the

Justice Department's desire for "roving wiretaps", designed to tap any

phone which a suspect might use. Including yours. If you pay U.S. taxes

you are paying for this directly.

Fortunately, the DTA does not prohibit the use of encryption products and

techniques such as Nautilus. See http://www.cdt.org/digi_tele/ for more

information.

We believe that the US Constitution entitles every citizen to use secure

communications to which only he or she controls the keys (see the First and

Fourth Amendments for more information). So we urge everybody to get a copy

of Nautilus *now* and start using it. Although we have many enhancements

planned for future versions that will make Nautilus better, the current

version is already reliable and provides everything necessary to protect

your privacy even if no further improvements are released.

For more info about the recently published FBI documents, see the Electronic

Privacy Information Center's web page on the subject at

http://www.epic.org/crypto/ban/fbi_dox/.

How Nautilus Works

Nautilus uses your computer's audio hardware to digitize and play back your

speech using one of several different speech compression algorithms built

into the program. It encrypts the compressed speech using your choice of

three different encryption functions, and transmits the encrypted packets

over your modem to your friend's computer. At the other end, the process is

reversed. The program is half-duplex; just hit a key to switch between

talking and listening.

Nautilus's encryption key is generated in one of two ways. Earlier versions

of Nautilus generated an encryption key from a shared secret passphrase that

you and your friend would choose together ahead of time. Beginning with

version 1.5, Nautilus generates the encryption key using the Diffie-Hellman

key exchange algorithm by default, although users may still revert to the

secret passphrase method if they prefer. Either way, once the key is agreed

on, the rest of the conversation is encrypted with your choice of secret key

cipher using the agreed-on key. Nautilus's three secret key ciphers are

(in order of decreasing speed) Blowfish, IDEA, and Triple DES. Blowfish is

the default.

Further details about Nautilus's cryptography can be found in the

Cryptography appendix (Appendix B).

----------------------------------------------------------------------------

2. System Requirements

Nautilus currently runs on two hardware platforms: IBM PC-compatibles and

desktop Sun Sparcstations (sun4c or sun4m architectures). For the IBM PC,

you will need to run MS-DOS, Linux, or possibly Solaris X86. For the

Sparcstations, you will need SunOS or Solaris. In each case, you need a 4800

bps modem or faster, or a direct connection to the Internet. See below for

info on specific systems.

Security Note

For most uses of Nautilus, the Unix/Linux versions provide good security.

But for highest security, you should use the MSDOS version of Nautilus,

under vanilla MSDOS. Do *not* use a DOS emulator under Linux or a DOS box

under Windows. This is because multitasking OS's such as Unix and Windows

use a "swap file" on disk to store data from tasks not currently running and

the OS's generally do not take special measures to zero out the swap file

after the process finishes. If the session key for your conversation is

stored in the swap file, an attacker with access to your machine may be able

to recover it later. There is nothing that an application program like

Nautilus can do about this without special help from the OS kernel which is

not always available.

Sun Sparcstation

Nautilus runs on desktop Sun Sparcstations under SunOS and Solaris using the

built-in audio hardware. If you are using a Sparcstation, you don't need to

worry further about your hardware configuration (except the modem).

IBM PC Compatible (MSDOS)

If you're using a PC, you need a 386/25 or faster processor (floating point

is not required) and a Soundblaster-compatible audio board. Boards that

provide Soundblaster emulation through software drivers rather than hardware

compatibility will not work. In particular, no sound boards that attach to

the computer via a PCMCIA slot or parallel port will work. This is because

Nautilus uses the Soundblaster's DMA functions and those functions work only

through the ISA bus.

The easiest way to find out if Nautilus can use particular sound card

without trying it is to ask whether it supports all versions of the computer

game DOOM. DOOM has the same audio hardware requirements as Nautilus, and is

popular enough that vendors and dealers usually will know if a card they

sell supports it. DOOM also has similar CPU requirements to Nautilus, so if

your computer can run DOOM with all of its sound effects (not just music),

it should be able to run Nautilus.

IBM PC Compatible (Linux)

Nautilus's CPU requirements under Linux are similar to its requirements

under MS-DOS, except that the current version requires floating point

hardware (387, 486dx, etc.). A future version may be able to run without

floating point, just like the DOS version, but don't hold your breath.

On the plus side, the Linux version of Nautilus uses Linux's /dev/dsp

interface to control the sound board. This means it can use any sound board

for which Linux has a driver. The Linux sound driver, called Unix Sound

System (formerly known as VoxWare), currently supports at least the

following sound cards: Soundblaster (and SBPro and SB16), Pro Audio Spectrum

16, Gravis Ultrasound. The Linux Sound HOWTO document also has a section on

sound cards supported by the sound driver under Linux.

A patch for the Linux kernel MAY allow a future version of Nautilus to

prevent itself from being swapped out. A copy of this patch is available

from

ftp://ftp.cc.gatech.edu/pub/linux/kernel/patches/misc/POSIX.4_locking.tar.gz.

This patch has not been tested by us, nor have we yet added code to Nautilus

which would take advantage of this feature.

Modem

You need a 14,400 bps or faster data connection to get the best speech

quality. However, Nautilus can also work at 9600 or 7200 bps with impaired

intelligibility, without extra cpu requirements. With a fast cpu, it can

work at 4800 bps with speech quality almost as good as the 14,400 bps coder.

7200 bps is slow enough to work over cellular phones much of the time,

especially if only one end of the connection is a cellular phone, and if the

cellular phone is stationary (i.e., not in a moving car). 4800 bps is even

more reliable. If you have trouble using Nautilus with a cellular modem, try

moving your phone to somewhere where the reception is better and keep the

phone stationary during the call.

We consider cellular phones to be an important use for Nautilus and will

continue to work on good quality coders that work at low bit rates.

Table of Features by Platform

IP Socket 7.2Kb/s 2.4Kb/s D-H Key

Support  Coders   Coder  Exchange

SunOS 4.1.x & 5.x [Image]     Y        Y       Y       Y

Linux 1.2 & Up            Y        Y       Y       Y

MS DOS w/ FPU             N        Y       Y       Y

MS DOS w/o FPU            N        Y       N       Y

Y          Support has been implemented and works

N          Support is not planned.

P          Support is in progress.

F          Support is planned but not in progress.

[Image] NOTE: SunOS 5.x (Solaris 2.x) support is available. There is a

problem in the audio driver that causes Nautilus to report ReadAudio()

failures. We're trying to fix this bug but it should not affect performance.

[Image] NOTE: There is a small bug with the audio drivers on some

SparcStations. The sampling rate of the audio device may be faster or slower

depending on the machine. This may manifest itself as one machine getting

farther and farther behind while playing audio. Eg: SparcStation 10 talking

to an IPX causes the IPX to fall behind. We're not sure which Sparcs have

this problem with respect to others but the SS10 to IPX is one example. We

also don't know if this will be a problem between Suns and PCs.

----------------------------------------------------------------------------

3. Quick Start

For the impatient, here is enough information about using Nautilus to let

you start using it immediately.

You and your friend must agree on whether you will originate the call (dial

out) or have your friend call you instead. The originator must know the

answerer's telephone number, for a direct modem-to-modem connection, or

their machine name or IP address, for an Internet connection. The answerer

should run the program first, so their computer is waiting for the

originator's computer to establish the connection.

Originating an outgoing call

To originate a call, first make sure the person you are calling is ready to

accept your call. He or she should have be running Nautilus in answer mode,

so that Nautilus is displaying "Waiting for incoming Nautilus connection" on

his or her screen.

Next, make sure your microphone is plugged into your computer and that it is

turned on. Then, run the following command for a direct modem-to-modem

connection under DOS:

C:\NAUTILUS> nautilus -o -p COM2 12025551212

Under Linux or Unix, you'd say

bash$ nautilus -o -p /dev/modem 12025551212

The "-o" option tells Nautilus to originate a call. The "-p" option tells

Nautilus to use where to look for your modem. The last argument on the

command line is the phone number you wish Nautilus to dial. Here is the

Internet version of the command line (Linux/Unix only):

% nautilus -o -i archie.sura.net

Again, the "-o" option tells Nautilus to originate a call. The "-i" option

tells Nautilus to use the Internet, rather than using a modem to dial the

answerer's modem directly (you and the answerer can still be connected to

the Internet via a modem, of course). The last argument on the command line

is the name of the answerer's machine.

Nautilus will now print some startup messages.

It is important that your microphone be plugged in and turned on before you

start up Nautilus, because Nautilus uses the audio energy coming into the

microphone to initialize the random number generator with which it creates

its session keys.

Nautilus does this sampling and initialization shortly after printing the

startup messages. Nautilus prints a rough estimate of the entropy found in

the sample, and warns you if it thinks the value is too low. If there is not

enough entropy, Nautilus's security may be compromised. This might happen if

your microphone input is turned off or shorted, or if there is not enough

sound reaching the microphone to give Nautilus the entropy it needs. The

amount of sound required depends on your microphone's sensitivity and also

on your computer's ambient noise level when you run Nautilus.

If Nautilus prints such a warning message, you will have the opportunity to

try again. Nautilus will record sound from the microphone for about 1/4 of

second starting immediately after you press the return key, so blow into the

microphone and continue blowing for a moment after pressing the return key.

If you are not able to get the audio entropy to be something larger than 0,

then it is likely that your microphone is off or not connected properly. If

you are able to get values other than 0, but not high enough for Nautilus to

accept them, it is likely that the recording level of your microphone input

is set too low.

Accepting an incoming call

To accept an incoming call, use the following:

C:\NAUTILUS> nautilus -a -p COM1

The "-a" option tells Nautilus to answer the phone the next time someone

calls, and the "-p COM1" option tells Nautilus to use COM1 to talk to the

modem.

Here is the Internet version of the command line:

% nautilus -a -i

The "-a" option tells Nautilus to wait for packets on the proper port to

establish the connection. The "-i" has the same meaning as for the

originator's command line above.

As with originate mode, you must provide some noise into the microphone, or

else Nautilus will give a warning that the audio entropy is too low.

A note on security

Nautilus normally initializes the encryption key for your conversation using

an ingenious technique called Diffie-Hellman key exchange after its

inventors, or DH for short. DH avoids the need for preagreed shared secrets

such as passphrases, but can be vulnerable to a "man in the middle" (MITM)

attack by an active eavesdropper. To defeat this attack, Nautilus prints an

authentication code (8 hex digits) as part of the key exchange. One user

simply reads this code (by voice) to the other user, who verifies it against

the code displayed on his screen. If the codes at both ends of the

conversation match, the conversation is secure.

In general, it is good to know what computers and routers on the internet

your conversation is going through, and the latency, or delay they cause.

Under Unix, the commands 'traceroute' and 'ping' are useful for finding such

information, although you should note that they use the ICMP protocol,

whereas Nautilus uses UDP. For more info, see (Appendix D).

Getting help

Typing "nautilus -h" will list all available program options along with

brief descriptions of their use. Also, running Nautilus with the "-v"

(verbose) option will print info on the screen about Nautilus's parameters

at startup time.

----------------------------------------------------------------------------

4. Compiling Nautilus

This section tells you how to compile Nautilus on MSDOS and Unix systems. We

distribute precompiled MSDOS binaries for user convenience, but for maximum

security, you should compile any cryptographic program yourself on your own

computer using a compiler that you know has not been tampered with.

To compile Nautilus, you'll need the RSAREF 2.0 library, available for free

for non-commercial use in the US and Canada from RSA Laboratories (see

ftp://www.rsa.com/pub for details).

I. MSDOS Systems

These instructions assume you have Microsoft C version 7.0 or later. The

makefile for Nautilus currently only works with Microsoft C, so if you have

some other compiler, you're on your own. If you do manage to get Nautilus to

build with some other compiler, please send us your changes and and we'll

include them in future releases.

1. Unpack the source distribution into a build directory.

2. Unpack the RSAREF 2.0 library and install it in a directory named

'rsaref' in the main build directory.

3. Change to the rsaref\install\dos subdirectory.

4. Copy the makefile in this directory to the parent directory.

5. Change to the rsaref\install subdirectory.

6. Use the Microsoft C 'nmake' program to build rsaref.lib

7. Copy the rsaref.lib library to the main build directory.

8. Change to the lpc10 subdirectory.

9. Copy makefile.dos to makefile.

10. Use the Microsoft C 'nmake' program to build lpc10.lib.

11. Change to the main build directory.

12. Copy makefile.dos to makefile.

13. Use the Microsoft C 'nmake' program to build nautilus.exe.

Alternatively, you can construct a project file for Nautilus and build it

from within the project manager. Just use the existing makefile to guide you

in the process of setting this up.

II. Unix Systems

Nautilus currently runs on Sun workstations with audio hardware running

SunOS 4.1.X and Solaris 2.X. It also runs on IBM PC-compatibles running

Linux. To build Nautilus, you will need an ANSI C compiler like gcc.

To build Nautilus under SunOS/Solaris/Linux, perform the following steps:

1. Unpack the source distribution into a build directory.

2. Unpack the RSAREF 2.0 library and install it in a directory named

'rsaref' in the main build directory.

3. Change to the rsaref/install/unix subdirectory.

4. Copy the makefile in this directory to the parent directory.

5. Change to the rsaref/install subdirectory.

6. Edit the makefile if necessary.

7. Type 'make rsaref.a' to build the rsaref library.

8. Change to the main build directory.

9. Link (or copy) makefile.unx to Makefile.

(i.e. 'ln -s makefile.unx Makefile' or 'cp makefile.unx Makefile')

10. Type 'make'. You will be presented with a list of platforms for which

the makefile knows how to build Nautilus. If you are building on a

different platform, you will need to edit the Makefile to include

support for your platform first (additional changes may also be

necessary to the source code).

11. Type 'make <platform>' where platform is one of the options listed

when you typed make in step 3 above. If all goes well, a nautilus

executable (and optionally a 'nuke' and 'unnuke' executable) will be

produced.

12. Type 'make install' to install the executable to /usr/local/bin

and the man page to /usr/local/man/man1.  Edit the makefile if you

want to install elsewhere.

----------------------------------------------------------------------------

5. Environment Variables

Nautilus allows you to pre-specify a passphrase using an environment

variable. Specifically, if one sets the NAUTILUS_PASSPHRASE variable before

running Nautilus, the program will not prompt the user for a passphrase and

will instead use the value of the environment variable. This enables

Nautilus to be called from a script if the user so desires. CAUTION: Be very

careful to destroy the NAUTILUS_PASSPHRASE variable when you are done with

your conversation. If your passphrase is revealed to an enemy that has

intercepted your transmission, the contents of your transmission can be

easily decoded.

On a PC running MSDOS, Nautilus requires a Soundblaster compatible sound

card. Installation software that comes with your Soundblaster tries to

configure your system so that the BLASTER environment variable is set.

Nautilus reads the contents of this environment variable in order to

determine the operating parameters for the card (I/O base address, IRQ,

DMA). If you are using another vendor's card (it must be hardware compatible

with the Soundblaster), that doesn't set the BLASTER environment variable,

you will need to set up a Nautilus configuration file which will tell the

program what parameters to use for your card. See the section on setting up

configuration files for information on how to do this.

----------------------------------------------------------------------------

6. Configuring Nautilus

Modem setup

Like your modem, Nautilus has two basic modes of operation. It can either

make a call (originate) or accept a call (answer). The "-o" option tells

Nautilus to originate a call, and the "-a" and "-A" commands tell Nautilus

to answer a call.

Nautilus assumes default settings for which serial port to use on your

computer, and at what speed to communicate with the modem (DTE speed). The

default serial port is COM1 and the default DTE speed is 19,200. We

recommend you use the default DTE speed of 19,200. Even though your modem

may be able to use compression to transfer some kinds of data faster than

its rated speed, it will not be able to compress the information sent to it

by Nautilus.

You also may need to turn off error correction (LAPM/MNP) on your modem,

as retransmission data may cause out-of-sequence packets.  See the example

nautilus.cfg for how to do this on a USRobotics.

If you have a modem that has a throughput of 19,200 bps or more

(e.g. V.32 turbo, V.FC, V.34, and HST), you may want to increase the DTE

speed to 38400, but make sure you have a high speed serial card (with a

16550A UART) before you do this.  If your modem is internal, you do not

have to worry about this, as internal modems emulate the UART.  However,

some problems have been reported with SoundBlaster-compatible soundcards

repeating speech when transmission is too fast.  If you experence this,

scaling back to 19200 should fix the problem.

IMPORTANT: You must configure your modem to use hardware flow control for

Nautilus to work. Unfortunately, the method for doing this varies from one

brand of modem to another, so Nautilus cannot automatically do it for you.

On the USR Courier or Sportster modem, sending the string "AT&F1" will set

hardware flow control among other things. For other modems, please consult

your modem manual.

In order to change the default port from COM1 to something else, use the

"-p" option followed by the name of the port you wish to use. The following

table describes which ports may be selected:

* COM1, at I/O address 0x3F8, using IRQ4

* COM2, at I/O address 0x2F8, using IRQ3

* COM3, at I/O address 0x3E8, using IRQ4

* COM4, at I/O address 0x2E8, using IRQ3

These are the "usual" COM port settings for PC-Compatible computers. Note

that in order to prevent interrupt conflicts, it is not possible to use both

COM1 and COM3 or COM2 and COM4 at the same time, unless you steal an IRQ

from something else (like an unused LPT2).

If you are using nonstandard addresses or IRQ's for your COM ports, you need

to customize Nautilus for your system using a configuration file. See the

"Setting up configuration files" section of this document for more info.

If you wish to change the default DTE speed on the command line from 19,200

to a higher value, use the "-s" option followed by the speed you wish to use.

Speech Coders

Nautilus currently includes four speech compression algorithms, also called

"coders". "Coder" is a speech processing term that has nothing to do with

cryptography; in this document we always refer to cryptographic coding as

"encryption" and cryptographic codes as "ciphers" to distinguish this

subject from speech coding.

The purpose of a speech coder is to convert raw digitized speech samples

into a compressed form that can be sent over channels with limited

bandwidth. Designing good speech coders is a highly technical subject

involving tradeoffs between CPU power, data bandwidth, sound quality, and

difficulty of implementation. Nautilus's current coders are fairly

sophisticated, though there is still room for improvement.

Generally, today's best-performing speech coders run on special-purpose

digital signal processing hardware since they need more CPU power than

typical PC's can provide. But as PC's get faster, we may be able to support

higher quality coders.

Three of Nautilus's currently running speech coders use a technique called

"switched prediction". Switched-predictive coders give a good tradeoff

between CPU speed requirements and speech quality at a given bit rate.

Nautilus has three SP coders with differing modem speed requirements, called

SP64, SP85, and SP124. The numbers refer to the bit rates used by the

coders: approximately 6400, 8500, and 12400 bits per second respectively,

with the higher bit-rate coders naturally sounding the best. Because of the

added data overhead imposed by Nautilus's two-way communication protocol,

the coders require modem connections of 7200, 9600, and 14,400 bps

respectively. 28,800 bps (v.34) modems are readily available at this time,

so speed is not an issue unless you have an older modem or a poor (or

cellular) connection.  9600 bps (v.32) is used by some older modems and is

used as a fallback speed for 14,400 bps modems when the phone lines are noisy. 7200 bps seems to be

about the highest speed that works reasonably well with v.32bis modems over

cellular telephones and even that speed requires a fairly good cellular

connection.

Nautilus's newest coder, LPC10, uses Linear Predictive Coding, and

compresses speech to 2400 bps with quality nearly as good as the SP124

coder. This comes at the expense of significantly more CPU time, although it

is able to run comfortably on a Sparc 2 or 486DX2-66 (slower machines may be

possible). It will not run quickly enough on a machine without an FPU.

Nautilus's overhead increases the bandwidth required to about 2800 bps for a

direct modem-to-modem connection, and additional IP overhead increases this

to about 4100 bps for an Internet connection. In either case, it should work

with a 4800 bps connection, which is the lowest fallback speed that you

normally ever get when you use a v.32 or faster modem. If your modem can

connect at all (including over cellular phones) and you can run LPC10,

Nautilus should work.

The LPC10 coder was obtained from URL:

ftp://ftp.super.org/pub/speech/ in file lpc10-1.0.tar.gz, and before that it

came from the NSA (yes, that NSA). It was modified somewhat so that it could

be included in Nautilus. The original and modified code is in Fortran, and

the modified sources are available from:

http://www.arl.wustl.edu/~jaf/lpc/

By default, Nautilus automatically selects the best sounding coder that will

run on both users' machines. Either user can override the default coder by

using the "-c" option. The option should be followed by the name of the coder

you want ("SP64", "SP85", "LPC10" or "SP124"). You can also use the

"-l coders" option to get a list of available coders (new ones may be added

in future releases).

See the bibliography file "nautilus.bib" in this directory for references

about speech coding and other subjects.

Ciphers

You may specify one of three encryption algorithms to secure your

conversation from would-be eavesdroppers. Triple-DES is the most proven of

the available ciphers, but also uses the most CPU. Blowfish is a relatively

new cipher designed by the author of _Applied Cryptography_, Bruce Schneier,

that is fast and shows promise of being strong (see

http://www.counterpane.com). Finally, IDEA is another cipher popularized by

PGP that has been included for completeness. Encryption can be specified

using the "-e" option and following it with the name of the specified cipher

or "NONE" to turn encryption off. Note that if the "-e" option is not

specified, Nautilus will use the Blowfish cipher by default. To get a list

of available ciphers, use the "-l ciphers" option.

Setting up configuration files

Nautilus can read in a number of parameters from a configuration file. The

parameters can be used to configure your serial port, sound card, and modem

initialization string. If you are using a sound card that does not set the

BLASTER environment variable (it must be soundblaster compatible), then you

will have to set up a configuration file in order to configure it for use

with your card.

The environment variable NAUTILUS_CONFIG_FILE specifies the pathname to a

Nautilus configuration file. If the environment variable doesn't exist,

Nautilus doesn't read a configuration file and uses its defaults. If it does

exist, Nautilus attempts to read the file specified and overrides the

defaults with the user-specified values.

The syntax of the configuration file is:

* White space is ignored

* All characters between a '#' and the end of a line are ignored

* Case is not significant

* An alpha character marks the beginning of an identifier

* Identifiers may contain alpha characters, numbers, and underscores only

* A number marks the beginning of a numeric constant. If the number

begins with "0x", it is interpreted as hexadecimal; otherwise, it is

considered decimal.

* A quote marks the beginning of a string constant

* Statements of the form: <identifier> = <constant> are the only legal

statements currently accepted by the parser

The following is a list of configuration variables and their types:

SPEED (numeric)      - DTE baud rate

PORT (string)        - port name (e.g. COM2)

MODEM_INIT (string)  - modem initialization string (don't include "AT")

SND_IO (numeric)     - i/o base address of sound card

SND_IRQ (numeric)    - irq number of sound card

SND_DMA (numeric)    - dma channel of sound card

MIC_SENS (string)    - mic sensitivity (one of "LOW", "MEDIUM", or "HIGH")

OUT_VOLUME (string)  - output volume (one of "LOW", "MEDIUM", or "HIGH")

COM1_IO (numeric)    - i/o base address of COM1 serial port

COM2_IO (numeric)    - i/o base address of COM2 serial port

COM3_IO (numeric)    - i/o base address of COM3 serial port

COM4_IO (numeric)    - i/o base address of COM4 serial port

COM1_IRQ (numeric)   - IRQ (interrupt number) of COM1 port

COM2_IRQ (numeric)   - IRQ (interrupt number) of COM2 port

COM3_IRQ (numeric)   - IRQ (interrupt number) of COM3 port

COM4_IRQ (numeric)   - IRQ (interrupt number) of COM4 port

UPGRADE_FILE (string)- filename to save upgrade instructions

if you try talking to an incompatible

version of Nautilus.  The instructions

are also shown on the screen.  If you

don't want them saved, set this to "".

Default setting is filename "UPGRADE".

An example configuration file follows:

#

# Example Nautilus Configuration File

#

PORT = "COM2"           # specify default serial port

SPEED = 38400           # specify default DTE speed

MODEM_INIT = "M0"       # turn modem speaker off

SND_IO = 0x220          # specify I/O Base address of sound card

SND_IRQ = 5             # specify IRQ for sound card

SND_DMA = 1             # specify DMA channel for sound card

MIC_SENS = "HIGH"       # specify high mic sensitivity

OUT_VOLUME = "LOW"      # specify low audio output level

----------------------------------------------------------------------------

7. Running Nautilus

When you first start Nautilus up, it will play the message "Welcome to

Nautilus" through your sound card. This lets you know that Nautilus is

properly configured to use your sound card. If you prefer not to hear this

message, you may specify the "-x" option on the command line.

Putting it all together:

To originate a call to another Nautilus user, type:

C:\NAUTILUS> nautilus -o -p com2 -s 38400 12025551212

This tells Nautilus use serial port COM2 with a DTE speed of 38400, to dial

the phone number "12025551212", and attempt to connect to an awaiting

Nautilus program at the other end.

To answer a call from another Nautilus user, type:

C:\NAUTILUS> nautilus -a

This tells Nautilus to configure the user's modem to answer the phone and

will then attempt to establish a connection to the remote Nautilus after the

modems have connected. The default port, COM1 is used as well as the default

DTE speed of 19,200, unless the .cfg file has been configured differently.

After Nautilus starts up and plays its welcome message, it will prompt you

to enter a passphrase (unless you have specified a passphrase via the

NAUTILUS_PASSPHRASE environment variable). At this time, you should type in

the passphrase you and your friend have previously agreed upon. You will be

asked to type the phrase twice to insure that you have not made a mistake.

If the two phrases don't match, you will be given the opportunity to try

again.

Once communication has been established, the originator will start out in

talk mode. Pressing a key will cause Nautilus to switch modes between

talking and listening. If the pass phrases typed by the users at both ends

do not exactly match, the users will be unable to communicate with each

other and will hear nothing but garbage coming out of their speakers.

An important option to remember when using Nautilus is "-h" (for "help").

Typing "nautilus -h" will cause Nautilus to print out a list of the commands

it understands and their syntax. Also, running Nautilus with the "-v"

(verbose) option will print info on the screen about Nautilus's parameters

at startup time.

IMPORTANT

As of this release (version 1.5d), Nautilus has been through five public beta

test releases and reviewed by several expert cryptographers, and nobody has

discovered any serious security problems with it. Some minor problems were

found and fixed during the beta tests. We now have more confidence in

Nautilus's security than we have for any comparable programs, mostly because

the other programs have not withstood public scrutiny of their source code

for as long (or at all).

The lpc10 coder has undergone some optimization for 1.5d and any feedback

on it is welcome.

Nonetheless, it's still possible that Nautilus has some security bugs that

haven't yet been discovered. Although we now consider Nautilus to no longer

be in "beta test", it has still not gotten nearly as much scrutiny as some

of the better known email encryption programs. For very high security

applications, we recommend avoiding placing your total faith in the security

of Nautilus or any other link in your system. Rather, take an in-depth

approach to security so that if any aspect of your communications chain is

compromised, your overall system is still not broken.

We continue to encourage cryptographers and users alike to examine and test

the program thoroughly, and *please* let us know if you find anything wrong.

As usual, although we'll try to fix any bugs reported to us, WE CANNOT BE

RESPONSIBLE FOR ANY ERRORS. Even if it uploads all your secret files to your

mother-in-law's computer in the middle of the night.

----------------------------------------------------------------------------

APPENDIX A: Getting Nautilus

FTP SITES

Nautilus is available in three different formats. As a DOS executable, it is

available as an archive in zip format along with it's associated

documentation. In source format, it is available as either a zip-ed archive,

or a gzip-compressed tar archive.

Various versions of Nautilus are available at the following FTP sites:

ftp://ftp.csn.org:/mpj/I_will_not_export/crypto_???????/voice/

This is an export controlled ftp site: read /mpj/README for information on

access.

ftp://ripem.msu.edu/pub/crypt/other/nautilus-phone-1.5a-source.tar.gz

ftp://ripem.msu.edu/pub/crypt/msdos/nautilus-phone-1.5a-source.zip

ftp://ripem.msu.edu/pub/crypt/msdos/nautilus-phone-1.5a-exe.zip

This is an export controlled ftp site: read /pub/crypt/GETTING_ACCESS for

information on access.

You may be able to find additional ftp sites using the "archie" ftp site

locating program. See http://www.earn.net/gnrt/archie.html for more info.

International Use

Sorry, but under current US law, Nautilus is legal for domestic use in the

US only. We don't like this law but have to abide by it while it is in

effect. Nautilus is distributed through export-restricted FTP sites for this

reason. Export it at your own risk.

Contacting The Developers

The Nautilus development team is now made up of Bill Dorsey, Paul Rubin,

Andy Fingerhut, Paul Kronenwetter, Bill Soley, Pat Mullarky, and David

Miller. To contact the developers, send email to <nautilus@lila.com>.

This announcement, and the source and executable distribution files, are all

signed with the following PGP public key. Please use it to check the

authenticity of the files and of any fixes we may post. You can also use it

to send us encrypted email if you want. We will try to keep such email

confidential, but cannot guarantee it. Encrypted messages will probably get

slower replies than unencrypted ones, because of the inconvenience of the

decryption process at our end. Please use unencrypted email unless you need

to send us something that really needs privacy.

pub  1024/FEFEDA09 1996/05/27 Nautilus Developers <nautilus@lila.com>

-----BEGIN PGP PUBLIC KEY BLOCK-----

Version: 2.6

mQCNAjGpL2QAAAEEALa5+7iMr/mTZc0+hpCY03y1cLSrZjEDCfGKDNr1dBvLj9O5

xnNagh1WWXhCccvzsva0Qs7+9H+IwJCCEqsOuA76aTf0wfLnJ6U5lwbcmB9jlter

D1bgVNRP20zWOP+MwU4uaUWib3l10RtDZM99kadlJNzT2PYq0vai5J3+/toJAAUR

tCdOYXV0aWx1cyBEZXZlbG9wZXJzIDxuYXV0aWx1c0BsaWxhLmNvbT6JAJUCBRAx

qTApg1x2TS1X7GUBATEnA/9+v4yEygNsxy05zYBLjKwI3E3FEUXMh7GjSfF+yp4b

iencWiErdsoiZl5fMzhAIzfrECaiHNFhSVokaXHjVL/SSlPfB/RF9LpekCxM4HKy

DGFxJlYfFelpvLXnhWy8tw3xMM8h40YinH/JFI7I4WLBAqFC7JwKXaoh5bbFwpVw

4A==

=YRNN

-----END PGP PUBLIC KEY BLOCK-----

Acknowledgements

Nautilus's 3DES implementation is adapted from code written by Richard

Outerbridge. The IDEA implementation is by Colin Plumb. The Blowfish

implementation is adapted from Bruce Schneier's original version, though

we've optimized it considerably. Dan Bernstein, Colin Plumb, and Bob Baldwin

examined the cryptographic protocols in the beta test versions and spotted

some errors as well as making other worthwhile suggestions. Andy Fingerhut

cleaned up and commented some of the speech compression code in version 1.0

and got the Linux port working again.

Nautilus Developer's PGP Key

The Nautilus Developer's Key should not be used for the moment. They will be

generating a new key shortly. In the meantime, if you must send sensitive

information, please use Bill Dorsey's key (dorsey@lila.com).

----------------------------------------------------------------------------

APPENDIX B: Cryptography and Security

A later version of this appendix will include advice on key selection and

exchange, other privacy issues, etc. For now, here is what we have.

Nautilus's Ciphers

Nautilus provides three ciphers: Triple DES, Blowfish, and IDEA. No

published methods exist that even make a scratch in the security of any of

these algorithms. Blowfish is the fastest and the most technically

straightforward, and is possibly the most secure of the three. But because

it relatively new, there is a higher potential of effective attacks against

it being discovered. Triple DES (abbreviated 3DES) is the most established

cipher and has withstood over two decades of intense scrutiny by

cryptographers. It is the safest choice, but it is slow. You might not be

able to use 3DES with Nautilus on a 386-based computer (a 486 or higher PC

or a Sparcstation should be no problem, but a 386DX/25 is known to not be

fast enough). IDEA, like Blowfish, is relatively new; it also incorporates

some unusual design principles that some cryptographers consider

questionable. It is faster than 3DES but slower than Blowfish. Also, IDEA is

patented; it is licensed for noncommercial use, but if you use Nautilus for

business purposes, selecting the IDEA cipher may be a patent infringement.

We include IDEA because it is used in the "PGP" email cryptography program;

many PGP users have come to trust it, so we offer them the option of using

it if that's what they want.

Blowfish is Nautilus's default cipher because of its speed, security,

traditional design, and freedom from patents. If you don't make your own

selection, you'll get Blowfish. But the choice is up to you.

A word about encryption and security

All reliable signs are that Nautilus's cryptographic algorithms (especially

3DES) are completely unbreakable in practice, even by three-letter agencies.

If someone does have a method that can break the ciphers, the method is only

useful if they keep existence a deep secret--otherwise you could change

ciphers and the attacker would have to start all over. And a fundamental

fact about any secret weapon is that every time you use it, the less secret

it becomes. So unless you're discussing matters so important that

cryptanalysis organizations would find it worth sacrificing part of the

usefulness of their most valuable tools, you can probably treat Nautilus's

ciphers as if they were totally impregnable.

Unfortunately, a totally impregnable cipher used in an insecure way is like

a totally impregnable steel bank vault door installed on a house with an

unlocked window. If someone wants to eavesdrop on your communications while

you're using Nautilus, cryptanalyzing the ciphers is probably the last thing

they would try. You have to take precautions against other types of attacks

as well.

First of all, although Nautilus's ciphers themselves are very secure and the

program itself has withstood some scrutiny over its past few releases, it

may still contain bugs. Nautilus's basic methods are simple and sound, but a

design oversight or programming error may have seriously weakened its

security. So far, no major security bugs in any Nautilus release have been

reported, but a few minor ones in the early beta test versions have been

found and fixed. We always announce such bugs when we learn of them.

Second, although Nautilus protects you from telephone line monitoring, it

does nothing to protect you from other methods of privacy invasion. Indeed,

history's most famous examples of electronic eavesdropping abuses--the

Watergate break-in and the FBI's recordings of Martin Luther King--were done

with room bugs rather than wiretaps. Secure telephones cannot protect your

privacy against hidden microphones in your bedroom or office.

A complete discussion of non-cryptographic security is beyond the scope of

this document, so we'll leave you on your own regarding technical

countermeasures against bugging (hint: no method is likely to be completely

effective, and we're skeptical of the motives of most people trying to sell

you expensive antibugging devices). One simple thing you can do is use a

headset microphone plugged into your sound board, rather than a loudspeaker

and handheld microphone. This will make it much more difficult for a room

bug to pick up the other end of the conversation (unless the other room is

also bugged).

Finally, although the encrypted signals Nautilus passes through your modem

cannot be decrypted without the secret key, your computer itself may leak

unencrypted information either through electromagnetic radiation or because

it has been tampered with. Displaying your passphrase on a CRT screen under

any circumstances is a potentially grave mistake, since CRT emissions can be

picked up from a considerable distance with the right equipment. With LCD

screens, the problem may not be as bad--but we don't know. This subject also

is beyond the scope of this document.

Traffic Analysis

While Nautilus protects the contents of your phone calls, it does not and

cannot do anything to conceal what time you make them or what numbers you

call. It is relatively easy for attackers to illegally obtain your phone

records, even if you have asked the phone company to place a passcode on

your account. This has actually happened to at least one Nautilus user who

was being illegally harassed by a private investigator. The PI called the

phone company claiming to be the person he was actually investigating and

requested info from "his" phone bill. When the phone company representative

asked for the passcode, the PI replied that he had just had eye surgery and

could not read the code. By a combination of whining and bullying, the PI

intimidated the phone company rep into giving him the info in violation of

phone company policy. The rep was mortified to find out afterwards that he

had been defrauded, but the damage was done.

If you need to conceal the phone numbers of the people you talk to as well

as the contents of the conversations, you must take further measures besides

using Nautilus. Make the phone calls between public phones in places you

cannot be predicted to be near. Don't call collect or use a phone company

credit card associated with your phone account; use coins, or use a prepaid

phone card that you bought with cash (7-11 stores sell these). Not many

public phones today have data ports, but some do; they are generally easiest

to find at airports. You can buy an acoustic coupler from Konexx that lets

you connect your modem to a phone with no data port, though the bit rate may

be low (possibly too low for Nautilus). If necessary, try several phones;

and you may have best results with a v.34-capable modem even though you are

only trying to get 7200 bps through the phone, because v.34 has potentially

better impaired-line performance than v.32bis.

----------------------------------------------------------------------------

APPENDIX C: Nautilus Key Exchange Protocol

Nautilus now offers two different key exchange protocols to select session

keys. This is done at the start of the Nautilus data conversation. Once the

session key is chosen, it is used to encrypt the digitized voice packets in

CFB mode for the rest of the conversation.

The two protocols offered are Diffie-Hellman key exchange and key exchange

based on a pre-agreed shared secret passphrase. The DH protocol is very

simple and exactly follows the standard references such as Applied

Cryptography. Here is a description of the protocol used to derive a session

key from pass phrases.

1. Users (originator and answerer) are prompted for a pass phrase.

2. Each side generates two different keys K1 and K2 by hashing the pass

phrase with two different salts S1 and S2.

3. Random number generator is seeded with input from the audio device.

4. Random number generator is used to generate the following random values

(called a key exchange structure) at each end:

KEY EXCHANGE STRUCTURE

+-------------------------+

| SESSION IV    (8 bytes) |

+-------------------------+ -

| TRANSMIT IV   (8 bytes) |  |

+-------------------------+  |

| RECEIVE IV    (8 bytes) |   >  Encrypted with selected cipher

+-------------------------+  |

| SESSION KEY  (56 bytes) |  |

+-------------------------+ -

5. The indicated portion of the key exchange structure is encrypted using

CFB mode. The "SESSION IV" is used as the IV for this operation. The

originator encrypts his keyexch structure with key K1, and the answerer

encrypts his keyexch structure with key K2.

6. The originator sends his encrypted key exchange structure to the

answerer. The answerer replieswith his own encrypted key exchange

structure.

7. Both sides decrypt the key exchange structure received from the other

side using K1 or K2 as appropriate.

8. Both sides XOR the unencrypted locally generated key exchange structure

with the decrypted key exchange structure received from the other side.

9. The transmit/receive IV and session key are extracted from the

structure resulting from the operation in step 8.

10. The session key is hashed into a key for the selected cipher.

* Note 1: "selected cipher" refers to one of Blowfish, 3DES, or IDEA. The

user can select one of these three ciphers at runtime. Blowfish is used

as a default if no selection is made.

* Note 2: The 56-byte length of the session key is somewhat arbitrary.

The only requirement I can think of for this key is that it be at least

as long as the key size of the cipher it is used with.

----------------------------------------------------------------------------

APPENDIX D: Man in the Middle (MITM) attack

By default, Nautilus sets up the encryption key for your conversation using

Diffie-Hellman (DH) key exchange. DH lets two users (call them Alice and

Bob) select a random shared encryption key without previously having any

shared secret information (such as passphrases) and without sending any

secrets over the communication channel. However, it can be vulnerable to a

"man-in-the-middle" (MITM) attack where an active eavesdropper (call her

Eve) can cut the communications channel in the middle and masquerade as

Alice to Bob, and as Bob to Alice. Now there are three participants and two

comm links:

Alice ---------  Eve --------- Bob

link 1         link 2

Eve sets up independent DH keys with Alice and Bob (call these keys KA and

KB). Eve decrypts Alice's remarks with KA, re-encrypts them with KB, and

relays them to Bob, and vice versa, meanwhile saving copies of the

unencrypted conversation to pass back to her superiors at the CIA. Alice and

Bob are blissfully unaware that they're not both using the same key. To

thwart the MITM attack, Nautilus prints a cryptographic checksum of the

agreed DH key on the users' screens. At the beginning of the conversation,

Alice should read this checksum (8 hex digits) to Bob, and Bob should verify

that it matches the checksum displayed on his screen. This authenticates

that both users are using the same key, unless the MITM is able to cut into

the conversation at just the right time and imitate Alice's voice well

enough to fool Bob into accepting the bogus checksum. This would require Eve

to be a person rather than a program, and she would need to be listening to

the conversation in real time and seizing control of it at just the right

moment, when Bob asks Alice to read the checksum (or if Alice asks Bob, Eve

has to imitate Bob's voice instead). The checksum approach, we feel, makes

the MITM attack impractical in almost any realistic situation.

For more info on Diffie-Hellman key exchange, the MITM attack, and almost

everything else cryptographic, see Applied Cryptography by Bruce Schneier,

now in its second edition. Errata for these editions is available at

http://www.counterpane.com.

----------------------------------------------------------------------------

"If you don't speak out, no one else will.  If you don't lead this

effort, no one else will.  For it's a task before all of us..."

-- Robert Fitzgerald Kennedy

----------------------------------------------------------------------------

"When cryptography is outlawed, only outlaws will have stegonography."

-- Joel McNamara

----------------------------------------------------------------------------