Date: Fri, 12 Oct 90 18:32:47 -0400

From: rosen@bu.edu (David B. Rosen)

To: Rich@rice.edu

In-Reply-To: Richard Murphey's message of Fri, 12 Oct 90 00:01:02 EST

Subject: translation of tek4014 to unix plot format

Here's the 4014 description I wrote awhile ago. It's based on a

description that came with the manual for a 4014 emulator, not on the

official description from tektronics. It also includes a description

of a resolution extension I proposed, which you might not want to

implement.

--

Tek 4014:

Pixel scale (pixels/length) same in x as y, i.e., the aspect ratio is unity.

1024 by 780 dots.

(Extended (EGM) resolution: 4096 by 3120.)

(My compatible but nonstandard extension: 16384 by 12480.)

Control codes:

GS  (ascii 29) -  enter graphics mode

US  (ascii 31) -  enter alpha mode

Escapes codes:

esc formfeed (ascii 12) - clear screen and enter alpha mode

esc `			- select normal line format

esc a			- select dotted line format

esc b			- select dot-dashed line format

esc c			- select short-dashed line format

esc d			- select long-dashed line format

The escape codes above are recognized from both graph and alpha state.

A sequence of N endpoints to be connected by drawing N-1 lines is given as:

<GS><endpoint1><endpoint2><endpoint3>... <endpointN>

A <US> should be put in every record after the last graphics sequence

so that if the terminal receives a broadcast message between any two

records, it will not be in graph state, at least in in operating

systems that do record-oriented i/o.

Each line should contain 78 or fewer characters so the system does not insert

a newline for software-wrapping terminals of 80 character width.

Each endpoint is a sequence of some or all of the following bytes (in order):

Bit:	 7(m.s.) 6	 5	 4	 3	 2	 1	 0(l.s.)

Byte:

A			 0	 1	-1y	-2y	-3y	-4y	-5y

EGM Extra      	-13y	 1	 1	-13x	-11y 	-12y	-11x	-12x

B		-14y	 1	 1	-6y	-7y	-8y	-9y	-10y

C			 0	 1	-1x	-2x	-3x	-4x	-5x

D		-14x	 1	 0	-6x	-7x	-8x	-9x	-10x

Bit -1Y is the most significant bit of the y-coordinate, etc.  Byte D is always

included, as it terminates the endpoint.  Bytes A and C may each be omitted if

their five bits are the same as those in the previous endpoint. If there  is no

change in y from the last point, then bytes A and B may be omitted.  If the EGM

byte is omitted, then the bits it represents are assumed zero.

A byte having 0,1 in bits 6,5 is A if it is before the first 1,1

(or if there is there is no 1,1);  otherwise it is C.

This scheme represents three compatible levels of resolution:

10 bits --  (bits -11, -12  missing on output, zero on input) --  Tek 4014/4010

12 bits -- (bits -13, -14 constant on output, const. or zero on input) -- EGM

14 bits --  My nonstandard extension

My extension is compatible when it is used to generate output that will be the

input to the standard schemes because the extra bits are simply ignored.  When

used to interpret input, my extension should be compatible with the output of

the standard schemes so long as the unused bits are constant.  In any case, the

worst that can happen is that some insignificant bits are random instead of

constant.