~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

~              README.DOC OF SCANKIT UTILITY                       ~

~                                                                  ~

~              September 25, 1992                                  ~

~                                                                  ~

~              Copyright (c) 1992 Mustek, Incorporated             ~

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

This file contains the following topics:

*  Running Scankit Utility in either Windows version 3.0 or 3.1

*  Editing the scanner device line

Before proceeding, it is important that you have completed the

following steps:

Step 1:  Installed the scanner interface card in your computer.

Step 2:  Connected the scanner to the interface card.

Step 3:  Run the Install program of Scankit Utility.

If you have not yet done any of the 3 steps, please refer to the

scanner Installation Guide for more details.

RUNNING SCANKIT UTILITY

~~~~~~~~~~~~~~~~~~~~~~~

1.  Start Windows by typing "win."

2.  Double-click the ScanKit Utility Icon(built	after installation) to

run	the Scankit Utility or,

2.1 Use	the Program Manager's File RUN command,	type:

"scanw"	(if you're using Windows 3.0/3.1)

Also include the DOS pathname.

(Examples: C:\SCANW\scanw  )

2.2 Click OK.

EDITING THE SCANNER DEVICE LINE

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

1.  INTRODUCTION

~~~~~~~~~~~~~~~~

For the computer to recognize that you have installed a scanner,

the CONFIG.SYS file should contain a device line that specifies

the scanner driver and the configuration (i.e., DMA, IRQ, and

I/O address settings) used by the scanner interface card.

TAKE NOTE THAT INSTALLING SCANKIT UTILITY WILL AUTOMATICALLY

UPDATE THE CONFIG.SYS FILE FOR THE SCANNER DEVICE LINE.

However, there are instances when you may need to manually

change the parameters in the scanner device line. Like, for

example, you have to change the IRQ setting to avoid a

hardware conflict.

To edit the CONFIG.SYS file, you will need a text editor.

If you have DOS 5.0, you could use EDIT.EXE. If you have DOS

version below 5.0, you could use EDLIN.COM. Under Windows, you

could also use the Windows Notepad. Please refer to your DOS

(or Windows) manual for steps on how to use these text editors.

Read carefully the following sections to obtain more information

on how to set the parameters in the scanner device line.

2.  THE SCANNER DEVICE LINE

~~~~~~~~~~~~~~~~~~~~~~~~~~~

The scanner device line should include five (5) items. These are:

*  Device driver

*  DMA setting

*  IRQ setting

*  Scanner type setting

*  I/O address setting

In the CONFIG.SYS file, the correct syntax for the scanner

device line is as follows:

DEVICE=[drive:\][path-directory\]driver-name	Dn In An Sn

where:

[drive:\][path-directory\]    is the drive and subdirectory

where the scanner driver is located

driver-name                   is the name of the scanner driver

(e.g., M1904.SYS / M1906.SYS )

Dn                            is the DMA setting of the interface card

(e.g., D3 for DMA3)

In                            is the IRQ setting of the interface card

(e.g., I5 for IRQ5)

An                            is the I/O address of the interface card

(e.g., A2AB for I/O address 2AB)

Sn                            is the scanner type

(e.g., S5 for GS-800 scanner)

3.  SETTING THE SCANNER DEVICE LINE PARAMETERS

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

When you edit the scanner device line, there are certain

precautions that you should take note of especially when

choosing the DMA, IRQ, and I/O address settings for the

scanner interface card.

Avoid hardware and software conflicts that can adversely

affect the performance of the scanner by selecting only the

available and unused DMA, IRQ, and I/O address settings. See

Appendix C of the Installation Guide for more information.

Use the following options for setting the scanner device line

parameters.

3.1  The Mustek scanner device driver  [driver-name]

The Mustek scanner device driver is found in the Scankit

Utility subdirectory. For the scanner to work, it is

important that the scanner driver is loaded properly when

you boot your computer. In the CONFIG.SYS file, make sure

the driver in the scanner device line has the correct path.

Driver-Name:    For all Mustek hand-held scanners:

------------    ----------------------------------

M1906.SYS                 CG-8000

M1904.SYS                 CG-6000

GS-800

GS-400

HT-800 TURBO

HT-105

Matador Models 4096, 256, 105

3.2  The DMA channel  [Dn]

When you assign the DMA setting for the scanner interface card,

make sure it is not used by any other device connected to

your computer. The available DMA channels for the

interface card are:

D1        for  DMA 1

D3        for  DMA 3 (recommended setting)

3.3  The IRQ level  [In]

When you assign the IRQ setting for the scanner interface card,

make sure it is not used by any other device connected to

your computer. The available IRQ levels for the interface

card are:

I3        for  IRQ 3

I5        for  IRQ 5  (recommended setting for XT bus)

I10       for  IRQ 10 (recommended setting for AT bus)

3.4  The Scanner type  [Sn]

S2        for  HT-105 / Matador 105

S4        for  CG-6000

S5        for  GS-800

S6        for  HT-800 TURBO

S7        for  GS-400 / Matador 256

S8        for  Matador  4096

S9        for CG-8000

3.5  The I/O Address  [An]

The scanner interface card has eight (8) available I/O

addresses and is equipped with a switch for selecting the

address setting.

Inspect the interface card closely. Printed on the circuit board

are the 8 available addresses. As with the DMA and IRQ settings,

select an I/O address that is not used by any other device

connected to your computer.

Take note also that the A parameter in the scanner device

line should match the scanner interface card's I/O address setting.

The 8 available I/O addresses are:

*  22B                          *  32B

*  26B                          *  36B

*  2AB (recommended setting)    *  3AB

*  2EB                          *  3EB

3.6  Examples of Scanner Device Lines

DEVICE=C:\SCANW\M1906.SYS D3 I5 S9 A2AB

In the example above, the scanner driver M1906.SYS is located

in subdirectory SCANW of drive C. The parameters used are

DMA 3, IRQ 5, and I/O address 2AB. The scanner model is the

CG-8000 color scanner.

DEVICE=C:\SCANW\M1904.SYS D3 I5 S4 A2AB

In the example above, the scanner driver M1904.SYS is located

in subdirectory SCANW of drive C. The parameters used are

DMA 3, IRQ 5, and I/O address 2AB. The scanner model is the

CG-6000 color scanner.

DEVICE=C:\SCANW\M1904.SYS D3 I5 S5 A3EB

In the example above, the scanner driver M1904.SYS is located

in subdirectory SCANW of drive C. The parameters used are

DMA 3, IRQ 5, and I/O address 3EB. The scanner model is the

GS-800 gray-scale scanner.

DEVICE=D:\SCANW\M1904.SYS D3 I10 S6 A26B

In the example above, the scanner driver M1904.SYS is located

in subdirectory SCANW of drive D. The parameters used are

DMA 3, IRQ 10, and I/O address 26B. The scanner model is the

HT-800 TURBO halftone scanner.

DEVICE=C:\SCANW\M1904.SYS D3 I5 S2 A3EB

In the example above, the scanner driver M1904.SYS is located

in subdirectory SCANW of drive C. The parameters used are

DMA 3, IRQ 5, and I/O address 3EB. The scanner model is either

the HT-105 or the Matador 105 halftone scanner.

AFTER YOU HAVE EDITED THE SCANNER DEVICE LINE IN THE

CONFIG.SYS FILE, REBOOT YOUR COMPUTER TO MAKE SURE THE

CONFIG.SYS FILE IS READ.

FOR PROBLEMS DUE TO HARDWARE AND/OR SOFTWARE CONFLICTS,

PLEASE SEE APPENDIX C OF THE SCANNER INSTALLATION GUIDE.